IEEE Trans. on  Instrum.  and  Meas.

Vol.45,  No.6,  p. 959,  Dec. 1996

Comments  on  "Modulo Time Plot ..."

The Modulo Time Plot, the use of reordered sample set is very useful for quick visual inspection of system performance [1]. This artifical-oversampling algorithm unrolls spectra, as well [2]. However, the question is how to reorder the samples.

To answer that, one must recall the conditions for uniform permutation of samples [3],[4]:

the sample set of a sinewave with  T = 1/f   period  and  sampling frequency  fs = 1/Dt

x[i] = cos(w.iDt) = cos(2p..i),           i = 0,1, ... N-1

is  periodic  only  if  the value of  numerical (normalized)  frequency

=  ,          1 £ m < (N/2)

where m prime to N, and N is the numerical period: x[i] = x[i+N]. This is the case of coherent sampling: N unique samples from m periods of the signal, N.Dt = m.T.  Such a raw sample set (m>1) can be reordered into single period by an index transformation

j = (J.i) mod N ,

where J is a unique integer and must be calculated from relation

(m.J) mod N  =  1 ,

i.e. J is multiplicative inverse of m.

Fig.1 shows a simple but informative example of transposition, where m/N=7/32, so that J = 23.

Some short notes on simulation: (1)  use DFT if N is not a  power-of-2; (2) n may be non integer and then it is the effective number of bits; (3) the common sinc interpolation formula [5] can be rearranged into a finite sum in case of periodic sample sets; (4) spectrum levels are in dBV; (5) with N=32, values of J for other possible frequencies:  m=3®J=11, 5®13, 9®25, 11®3, 13®5, 15®15; (6) since the numerical frequency is a ratio of small integers, there is a massive superposition of the aliased harmonics.

Simulation parameters may be easily modified, i.e. in [1] m/N=1501/4096, so that J = 2677;  or,  with reindexing from zero in [6, fig.2]  the numerical  frequency  is  m/N=4/19, so that J = 5.

The technique of the uniform permutation can be used for other desirable waveform or spectrum modifications.

Zsolt PÁPAY

Dept. of  Telecommunications

Technical Univ. of Budapest,  HUNGARY

References

[1]  F. H. Irons and D. M. Hummels, "The Modulo Time Plot - A Useful Data Acquisition Diagnostic Tool,"  IEEE Trans. on Instrum. and Meas.,  vol. IM-45, no. 3, pp. 734 - 738,  June 1996.

[2] S. Renaud, "Algorithm unrolls spectra,"  EDN October 13, 1988 (pp. 246 , 248).

[3] S.C Kak and N.S. Jayant, " On Speech Encryption Using Waveform Scrambling," Bell System Tech. J., Vol. 56, no. 5, pp. 781 - 808, May - June 1977.

[4] D. L. Duttweiler and D. G. Messerschmitt, "Analysis of Digitally Generated Sinusoids with Application to A/D and D/A Converter Testing," IEEE Trans. on Communications,  vol. COM-26,  no. 5,  pp. 669 - 675,  May 1978.

[5]  R. J. Marks II,  Introduction to Shannon Sampling and Interpolation Theory. New York: Springer - Verlag, 1991 (p. 33,  p. 89)

[6] Std. 1057-1994, "Standard for Digitizing Waveform Recorders," IEEE Std. 1057 June 14  1994 (p. 29)

[7]   Mathcad 5.0,   Mathsoft Inc.

*********************************************************************

Note:  The typographical gremlin(s)  made  an mistake  in the article:

an  important part of  Fig.1  (® the reordering of raw samples)  was  omitted.

See:  “Correction to ...”  IEEE Trans. on IM, Vol.46, No.3, p.739, June 1997

*********************************************************************